Melexis SAE J2602 Firmware API

Programmer’s Reference Manual

SWCC

14/09/2018

Contents

1. About this document	4

1.1. Purpose	4

1.2. Target Audience	4

1.3. Glossary	4

1.4. References	4

2. General description	5

3. Conventions and Types Definitions	5

3.1. Inherited data types	5

3.2. Specific for J2602	5

3.2.1. l_J2602ResetResp_t	5

4. API Commands and events	5

4.1. Inherited commands and events	6

4.2. Obsolete commands end events	6

4.3. Constants	6

4.3.1. ML_NUMBER_OF_DYNAMIC_MESSAGES	6

4.4. Initialization and configuration	6

4.4.1. l_ifc_init_sl1	6

4.4.2. ld_read_configuration	7

4.4.3. ld_set_configuration	8

4.5. Events raised to the application level	9

SAE J2602 Status Byte handling	9

4.5.1. mlu_SAE_SetStatusApiInfo	9

4.5.2. mlu_SAE_MessageReceived	10

4.5.3. mlu_SAE_DataTransmitted	10

4.5.4. mlu_SAE_BroadcastReceived (not implemented yet)	11

4.5.5. mlu_SAE_Reset	12

4.5.6. ld_serial_number_callout	13

4.5.7. ld_read_by_id_callout	13

| Date | Version | Name | Comments |
| --- | --- | --- | --- |
| Apr 27, 2007 | 1.4 | Yevheniy Soloshenko | Outlined together with Platform 1.4 |
| Aug 16, 2007 | 1.5 | Yevheniy Soloshenko | 1. Added ML_TARGETED_RESET / ML_BROADCAST_RESET parameter to mlu_SAE_TargetedReset notification. Notification is renamed to mlu_SAE_Reset 2. Introduced mlu_SAE_DataTransmitted notification 3. Added notes on number of automatically assigned message ID’s (NUM_MSG) in ml_SAE_LinInit 4. Added ld_serial_number_callout function to get serial number from user application |
| Feb 9, 2009 | 1.5.3 | Yevheniy Soloshenko | Function mlu_SAE_BroadcastReceived is marked as not implemented |
| Oct 10, 2016 | 1.6 | Pavlo Sapsai | Added full J2602 Melexis LIN-API into platform, and ldf-dependent file is generates by using of ldf_nodegen utility. Added function mlu_SAE_SetStatusApiInfo instead of mlu_SAE_DataRequest. Removed function ml_SAE_LinInitModule. |
| Dec 27, 2016 | 1.6.1 | Pavlo Sapsai | Added functions ld_diagMasterReq_callout, ld_read_by_id_callout, ld_set_configuration and ld_read_configuration. Added attribute weak to mlu_SAE_MessageReceived and mlu_SAE_DataTransmitted. |
| Mar 12, 2018 | 2.0.0 | Greg Moroz | Removed ld_diagSlaveRes_callout(), ld_diagMasterReq_callout(), mlu_TL_TimeoutStart. Updated the mlu_SAE_Reset() interface. |

# About this document

## Purpose

This document describes the Melexis SAE J2602 Firmware API implementation based upon the LIN2.0 [1] and SAE J2602-1 specifications [2]. Simple usage examples are given for each command and event function.

## Target Audience

The target audience of this document includes, but is not limited to software and application engineers who design LIN application firmware.

## Glossary

Table 1 explains the abbreviations used in this document.

Table 1: Abbreviations used in this document

| Abbreviation | Description |
| --- | --- |
| API | Application Programming Interface |
| DNN | Device Node Number |
| DUT | Device Under Test |
| EMC | Electromagnetic compatibility |
| ESD | Electrostatic Discharge |
| FW | Firmware |
| ID | Identifier |
| IUT | Implementation Under Test |
| LIN | Local Interconnect Network |
| LIN Firmware | Consists of MLX4 code and LIN API code |
| LIN Module | Consists of MLX4 microcontroller and physical LIN cell |
| NAD | Node Address for Diagnostics |
| SAE | Society of Automotive Engineers |

## References

- LIN 2.0 protocol specification

- SAE J2602-1 Surface Vehicle Recommended Practice – LIN Network for Vehicle Applications, 2005-09

- Melexis LIN Firmware API - Programmer's Reference Manual

# General description

The Melexis SAE J2602 Firmware is based upon the Melexis LIN API commands and events [3].

The main features of Melexis SAE J2602 Firmware are:

- LIN protocol errors are detected and reported via SAE J2602 Status Byte

- User application message ID’s are assigned according to DNN/NAD

- Support of LIN 2.0 configuration/diagnostic messages Assign NAD, Assign FrameID, Read by Identifier

- Support of SAE DNN broadcast messages

- Support of Targeted/Broadcast Reset message

# Conventions and Types Definitions

## Inherited data types

The following data types defined in Melexis LIN Firmware API [3] are inherited by SAE J2602 API:

- l_u8

- l_u16

- l_bool

- ml_Status

- ml_MessageID

## Specific for J2602

### l_J2602ResetResp_t

ml_ResetStatus is a basic integer type sufficient to represent the status of a mlu_SAE_Reset command. Possible values are shown in the table below.

| Value | Meaning |
| --- | --- |
| J2602ResetNoResponse | No response required |
| J2602ResetPosTarReset | Positive answer is expected |
| J2602ResetNegTarReset | Negative answer is expected |

# API Commands and events

All API commands and events of the LIN Firmware API [3] apply with the following extensions and changes.

## Inherited commands and events

The following commands and events defined in Melexis LIN Firmware API [3] are inherited by SAE J2602 API:

- ml_Connect

- ml_Disconnect

- ml_GetState

- ml_GotoSleep

- ml_WakeUp

- ml_DataReady

- ml_DiscardFrame

- ml_AutoAddressingConfig

- mlu_AutoAddressingStep

## Obsolete commands end events

The following commands and events defined in Melexis LIN Firmware API [3] are obsolete in Melexis SAE J2602 API:

- ml_SetOptions

- ml_InitLinModule

- ml_SetBaudRate

- ml_AssignFrameToMessageID

- ml_EnableMessage

- ml_DisableMessage

- mlu_DataRequest (use mlu_SAE_SetStatusApiInfo to set application-specific 5 bit of StatusByte)

- mlu_DataTransmitted (use mlu_SAE_DataTransmitted instead)

- mlu_ErrorDetected

- mlu_MessageReceived (use mlu_SAE_MessageReceived instead)

## Constants

### ML_NUMBER_OF_DYNAMIC_MESSAGES

ML_NUMBER_OF_DYNAMIC_MESSAGES constant defines number of message per node. This constant is defined in lin_signals.h file and should be in powers of 2, i.e. 4, 8 or 16. During initialization ML_NUMBER_OF_DYNAMIC_MESSAGES messages are automatically assigned to the node [2]. Frame identifiers of the messages are based on the DNN/NAD value.

## Initialization and configuration

### l_ifc_init_sl1

Syntax

void l_ifc_init_sl1 (void);

Description

Initializes the LIN module according to SAE J2602, i.e. sets baudrate, module’s parameters, initial NAD and node configuration. The number of automatically assigned message ID’s is defined by ML_NUMBER_OF_DYNAMIC_MESSAGES constant (could be 4, 8 or 16).

During initialization ML_NUMBER_OF_DYNAMIC_MESSAGES messages are automatically assigned to the node. Identifiers of the messages are based on the DNN/NAD value.

After initialization the LIN module shall be connected to the LIN bus using l_ifc_connect_sl1 command.

Initial NAD will be set at start-up. According to the SAE specification, the NAD shall be in range 0x60…0x6F, where the lower nibble of the J2602 NAD contains a four bit Device Node Number (DNN). SAE NAD 0x6F should be used for uninitialized nodes, which NAD will be changed at runtime, e.g. Software Programmable NAD. If SAE NAD 0x6E is used, message IDs will not be automatically assigned for this node and shall be assigned via LIN configuration messages.

Preconditions

The command shall be executed before any other LIN API’s command. The command MUST be called in disconnected state of LIN module.

Return Value

None

Note

This command replaces ml_InitLinModule.

This command obsoletes ml_SetOptions.

Example

int main(void)

{

l_ifc_init_sl1();

(void)l_ifc_connect_sl1();

/* ...*/

}

See also

ml_Connect [3]

### ld_read_configuration

Syntax

l_u8 ld_read_configuration (l_ifc_handle iii, l_u8* const data,

l_u8* const length);

Description

Read the node configuration. This call will serialize the current configuration and copy it to the area (data pointer) provided by the application. After the call is finished the application is responsible to store the data in appropriate memory.

The data is a pointer to the data buffer provided by application, length is a pointer to the length of the buffer.

Preconditions

This function could be called from the LIN-API when the HAS_SAVE_CONFIGURATION_FUNCTIONS defined to 1 in lin_signals.h.

Return Value

It returns l_u8 status: LD_READ_OK if the service was successful or LD_LENGTH_TOO_SHORT if the configuration size is greater than the length.

See also

ld_set_configuration [3]

### ld_set_configuration

Syntax

l_u8 ld_set_configuration (l_ifc_handle iii, const l_u8* const data,

l_u16 length);

Description

Set the node configuration. The function will configure the NAD and the PIDs according to the configuration given by data. The intended usage is to restore a saved configuration or set an initial configuration (e.g. coded by I/O pins).

The data is a pointer to the data buffer provided by application, length is a pointer to the length of the buffer.

Preconditions

This function could be called from the LIN-API when the HAS_SAVE_CONFIGURATION_FUNCTIONS defined to 1 in lin_signals.h.

Return Value

It returns l_u8 status: LD_READ_OK if the service was successful, LD_LENGTH_NOT_CORRECT if the required size of the configuration is not equal to the given length or LD_DATA_ERROR if the set of configuration could not be made.

See also

ld_read_configuration [3]

## Events raised to the application level

The below notification functions are called by the LIN ISR, and hence the processing in these functions should be kept as short as possible as other interrupts may be disabled or their ISR delayed until the LIN ISR has returned.

## SAE J2602 Status Byte handling

### mlu_SAE_SetStatusApiInfo

Syntax

l_u8 mlu_SAE_SetStatusApiInfo (ml_MessageID midx);

Description

This command fill into SAE J2602 Status Byte 5 bit of apiInfo (Application Information Field) using Bits4-0 and Error Field (Bits 7-5) will be filled automatically inside the J2602 LIN-API. Then it inserts the SAE J2602 Status byte as the first byte of the LinFrameDataBuffer. After that, the LIN Module is signaled that the data in the LinFrameDataBuffer is ready to be transmitted on the LIN bus.

Return Value

Returns apiInfo byte.

Note

This command extends the mlu_DataRequest.

Example

l_u8 mlu_SAE_SetStatusApiInfo (ml_MessageID midx)

{

l_u8 SAE_apinfo;

switch (midx) {

case msg_PUB1:                      /*  TX, 2 bytes */

l_u8_wr_PUB1_B1(0xFF);

SAE_apinfo = 1u << 4;

break;

case msg_PUB2:                      /*  TX, 2 bytes */

l_u8_wr_PUB2_B1(0x11);

SAE_apinfo = 1u << 3;

break;

default:

SAE_apinfo = 0u;

break;

}

return SAE_apinfo;

}

See also

mlu_SAE_MessageReceived

### mlu_SAE_MessageReceived

Syntax

void mlu_SAE_MessageReceived (ml_MessageID midx);

Description

This notification is called when the data of a relevant frame is successfully received. Message Index midx indicates which message has been received. This call has to be done as soon as possible so that the next message can be stored. In LIN-API this function has dummy implementation with weak attribute and could be redefined in the customer application.

Return Value

None

Note

This command extends the mlu_MessageReceived.

Example

void mlu_SAE_MessageReceived (ml_MessageID midx) {

switch (midx) {

case SUB1_MIDX:

if (  (LinFrameDataBuffer[0] == 'M')

&& (LinFrameDataBuffer[1] == 'E')

&& (LinFrameDataBuffer[2] == 'L')

&& (LinFrameDataBuffer[3] == 'E')

&& (LinFrameDataBuffer[4] == 'X')

&& (LinFrameDataBuffer[5] == 'I')

&& (LinFrameDataBuffer[6] == 'S'))

{

/* Placeholder for the Frame Hamdler ... */

}

break;

default:

break;

}

}

See also

ml_ReleaseBuffer [3]

### mlu_SAE_DataTransmitted

Syntax

void mlu_SAE_DataTransmitted (void);

Description

This notification is called when the frame published by our slave node is successfully transmitted over the LIN bus. In LIN-API this function has dummy implementation with weak attribute and could be redefined in the customer application.

Return Value

None

Note

This command extends the mlu_DataTransmitted, but mlu_SAE_DataTransmitted notification is not sent for Slave Response diagnostic frames (0x3D)

Example

void mlu_SAE_DataTransmitted (void)

{

/*

* This notification can be used to notify the application

* about the successful transmitting of the frame

*/

}

See also

ml_DataReady [3]

mlu_DataTransmitted [3]

### mlu_SAE_BroadcastReceived (not implemented yet)

Syntax

void mlu_SAE_BroadcastReceived (

ml_MessageID midx,

ml_uint8 *data);

Description

This notification is called when the data of a relevant DNN broadcast message is successfully received. Message Index midx indicates which DNN broadcast message has been received. Depending on the node’s current NAD, midx could be DNN_BROADCAST_38H / DNN_BROADCAST_3AH or DNN_BROADCAST_39H / DNN_BROADCAST_3BH.

Argument data points to the beginning of the broadcast data which is addressed to the node. The number of bytes in the broadcast message is equal to the number of node’s messages ID’s divided by 4, i.e. NUM_MSG / 4.

Once the broadcast data is read, the ml_ReleaseBuffer command should be used as in the example below. This call has to be done as soon as possible so that the next message can be stored.

Return Value

None

Example

void mlu_SAE_BroadcastReceived (ml_MessageID midx, l_u8 *data)

{

if ((DNN_BROADCAST_38H == midx) || (DNN_BROADCAST_39H == midx)){

if (data[0] == 0xB1) {

/* some action ...*/        }

}

else {  /* DNN_BROADCAST_3AH or DNN_BROADCAST_3BH then */

if (data[0] == 0xB2) {

/* some action ...*/

}

}

ml_ReleaseBuffer();

}

Implementation note

Not implemented yet

See also

ml_ReleaseBuffer [3]

### mlu_SAE_Reset

Syntax

l_J2602ResetResp_t mlu_SAE_Reset(void);

Description

This notification is called when an SAE Reset frame is received. The application is responsible to parse the LinFrameDataBuffer to return appropriate value.

Return Value

Returns l_J2602ResetResp_t.

Example

l_J2602ResetResp_t mlu_SAE_Reset (void) {

l_J2602ResetResp_t retVal;

if (LinFrameDataBuffer[0] == 0x7F) {

/* Broadcast reset: no response */

retVal = J2602ResetNoResponse;

} else {

/* Targeted reset: positive or negative response */

retVal = J2602ResetPosTarReset; /* positive response */

}

return retVal;

}

### ld_serial_number_callout

Syntax

void ld_serial_number_callout (l_u8 data[4]);

Description

This callout function is called by the LIN driver to retrieve the serial number from the user application. Serial number is requested by the LIN driver to prepare a response to master’s “Read by identifier” request with id=1 (serial number).

The data array of 4 bytes must be used by the user application to transfer the serial number.

Preconditions

This callout function is called from the LIN driver when the “Read by identified” request with id=0 (serial number) is received from the master and if HAS_SERIAL_NUMBER_CALLOUT defined to 1 value in lin_signals.h.

Return Value

None.

Example

void ld_serial_number_callout (l_u8 data[4])

{

data[0] = (l_u8)ML_SERIAL_NUMBER;           /* LSB */

data[1] = (l_u8)(ML_SERIAL_NUMBER >> 8);

data[2] = (l_u8)(ML_SERIAL_NUMBER >> 16);

data[3] = (l_u8)(ML_SERIAL_NUMBER >> 24);   /* MSB */

}

See also

ld_read_by_id_callout [3]

### ld_read_by_id_callout

Syntax

l_u8 ld_read_by_id_callout (l_ifc_handle iii, l_u8 id, l_u8* pci, l_u8* data);

Description

User defined read-by-identifier request. This callout is used when the master node transmits a read by identifier request with an identifier in the user defined area. The slave node application will be called from the driver when such request is received.

The id is an identifier in the user defined area (32 to 63),  the data is and array (of 5 bytes maximum), which should be used by the user application to transfer the data over the response frame, pci is a pointer for setting PCI of the response and it defines data length plus one.

Preconditions

This callout function is called from the LIN driver when the “Read by identified” request with id>1 is received from the master and HAS_READ_BY_ID_CALLOUT defined to 1 in lin_signals.h.

Return Value

It returns l_u8 Status: LD_NEGATIVE_RESPONSE if the slave node will respond with a negative response,

LD_POSTIVE_RESPONSE if the slave node will setup a positive response or LD_NO_RESPONSE if the slave node will not answer.

Note

The function interface differs from the Standard definition because of additional pci parameter.

Example

l_u8 ld_read_by_id_callout (l_ifc_handle iii, l_u8 id, l_u8* data)

{

(void) iii;

/*

* id values are in range from 0x20 to 0x3F (32 to 63),

* their meanings are application dependent

*/

if (id == 0x20)

{

data[0] = 0xAA;

data[1] = 0xBB;

data[2] = 0xCC;

data[3] = 0xDD;

data[4] = 0xEE;

return LD_POSITIVE_RESPONSE;

}

else

{

return LD_NEGATIVE_RESPONSE;

}

}

See also

ld_serial_number_callout [3]