Melexis Standard LIN API

Programmer’s Reference Manual

SWCC

March 15, 2018

# Abstract

This document is a reference manual for programmers to integrate the LIN Module in their applications according to the Standard LIN Application Program Interface in the C programming language as specified in the LIN 2.0 and LIN 2.1 specifications.

# History records

| Date | Version | Name | Comments |
| --- | --- | --- | --- |
| Jul 17, 2007 | 0.1 | Yevheniy Soloshenko | Document creation |
| Jul 19, 2007 | 0.2 | Yevheniy Soloshenko | Split in external API and SW Specification |
| Aug 16, 2007 | 0.3 | Yevheniy Soloshenko | Added ld_serial_number_callout function |
| Oct 4, 2007 | 1.5 | Yevheniy Soloshenko | Release Transport layer not supported |
| Nov 15, 2007 | 1.5.1 | Helen Salenko | LIN 2.1 API description added |
| Mar 27, 2008 | 1.5.2 | Helen Salenko | LIN Transport layer supported |
| Jul 10, 2008 | 1.5.2.1 | Yevheniy Soloshenko | Changed all references to “System mode” of MLX16 CPU to “Protected mode” as per MLX16 documentation |
| Feb 10, 2008 | 1.5.3 | Gjalt de Jong | Added ld_ready_by_id_callout function Added ml_DiagClearPendingResponse function Added mlu_TL_Timeoutstart function |
| Aug 16, 2010 | 1.6.0 | Gjalt de Jong | Added ld_functional_request_callout function Added notes wrt optional services Added reference to Conditional Change NAD service for ld_serial_number_callout and ld_read_by_id_callout |
| Aug 16, 2010 | 1.7.0 | Yevheniy Soloshenko | Added l_ifc_sleep_entered_callout function [request PLTF-579] https://extranet.melexis.com/jira/browse/PLTF-579 |
| Jan 18, 2017 | 1.8.0 | Grygoriy Moroz | Review; style update with new corporate branding |
| Sep 06, 2017 | 1.9.0 | Grygoriy Moroz | Added ld_diagSlaveRes_callout() description |
| Mar 12, 2018 | 2.0.0 | Grygoriy Moroz | Removed ld_diagSlaveRes_callout(), ld_diagMasterReq_callout(), mlu_TL_TimeoutStart Removed non-relevant information due to refactored MLX LIN API usage. |

# Contents

1. About this document	5

1.1. Purpose	5

1.2. Target Audience	5

1.3. Glossary	5

1.4. References	5

2. General description	6

3. Conventions and Type Definitions	7

3.1. LIN Standard API data types	7

3.1.1. Typedefs	7

3.1.2. Enumerations	7

3.1.2.1. l_ifc_handle	7

3.1.2.2. SlaveResponse_t	7

3.1.3. Data structures	8

3.1.3.1. l_s_IfcStatus_t	8

3.1.3.2. l_s_DiagFrame_t	8

3.2. Constants	8

4. API Commands and events	9

4.1. Driver and cluster management	9

4.1.1. l_sys_init	9

4.2. Signal interaction	9

4.2.1. Scalar signal read	9

4.2.2. Scalar signal write	10

4.2.3. Byte array read	10

4.2.4. Byte array write	11

4.3. Notification	11

4.3.1. l_flg_tst	12

4.3.2. l_flg_clr	12

4.4. Schedule management	12

4.5. Interface management	13

4.5.1. Calls common for LIN 2.0, LIN 2.1 and LIN 2.2 API versions	13

4.5.1.1. l_ifc_init	13

4.5.1.2. l_ifc_goto_sleep	13

4.5.1.3. l_ifc_wake_up	13

4.5.1.4. l_ifc_ioctl	14

4.5.1.5. l_ifc_read_status	14

4.5.2. Calls specific for LIN 2.0 only	15

4.5.2.1. l_ifc_connect	15

4.5.2.2. l_ifc_disconnect	15

4.5.3. Calls specific for Melexis LIN API implementation	16

4.5.3.1. l_ifc_sleep_entered_callout	16

4.6. User provided call-outs	16

4.6.1. l_sys_irq_disable	16

4.6.2. l_sys_irq_restore	17

4.7. Network configuration and identification	17

4.7.1. Calls common for LIN 2.0, LIN 2.1 and LIN 2.2 API versions	17

4.7.1.1. ld_conditional change_NAD	17

4.7.1.2. ld_read_by_id	17

4.7.1.3. ld_assign_nad	17

4.7.1.4. ld_read_by_id_callout	18

4.7.2. Calls specific for LIN 2.0 only	19

4.7.2.1. ld_is_ready	19

4.7.2.2. ld_check_response	19

4.7.2.3. ld_assign_frame_id	19

4.7.3. Calls specific for LIN 2.1 only	19

4.7.3.1. ld_is_ready	19

4.7.3.2. ld_check_response	19

4.7.3.3. ld_assign_frame_id_range	19

4.7.3.4. ld_save_configuration	20

4.7.3.5. ld_read_configuration	20

4.7.3.6. ld_set_configuration	20

4.7.4. Calls specific for Melexis LIN API implementation	21

4.7.4.1. ml_DiagClearPendingResponse	21

4.7.4.2. ld_serial_number_callout	21

5. Disclaimer	22

# About this document

## Purpose

This document describes the Melexis Standard LIN Application Program Interface for the Signal Interaction Layer based upon LIN2.0 [1], LIN2.1 [2]and LIN2.2 [3]. It includes the Core API and the Node Configuration and Identification as specified in [1][2][3].

Simple usage examples are given for each command and event function.

## Target Audience

The target audience of this document includes, but is not limited to software and application engineers who design LIN application firmware.

## Glossary

Table 1 explains the abbreviations used in this document.

| Abbreviation | Description |
| --- | --- |
| API | Application Programming Interface |
| FW | Firmware |
| ID | Identifier |
| LIN | Local Interconnect Network |
| LIN Firmware | Consists of MLX4 code and LIN API code |
| LIN Module | Consists of MLX4 microcontroller and physical LIN cell |
| NAD | Node Address for Diagnostics |

Table 1: Abbreviations used in this document

## References

- LIN 2.0 protocol specification

- LIN Specification package. Revision 2.1

- LIN Specification package. Revision 2.

- LDF LIN Node driver generation utility – User Manual

# General description

The LIN firmware supports the data link layer of LIN specification 2.0, 2.1 and 2.2.

Supported features include:

- Frame protocol processing

- Support for unconditional sporadic and event-triggered frames

- Support for error and status management

- Support for interface management:

  - Goto-sleep command and wakeup request

  - Enter sleep mode on inactive LIN bus

- Node configuration services:

  - Assign frame ID / (Assign frame ID range)

  - Assign NAD

  - Conditional change NAD

  - Save Configuration

  - Read-by-identifier

- Frame slot duration checking

- Automated LDF-dependent LIN API part generation [5]

Melexis Standard LIN API implements the Core API, Node Configuration and Identification commands as specified in the LIN 2.0 LIN 2.1 and LIN 2.2 specifications for a LIN slave [1][2][3].

Frame handler

(MLX4 LIN FW)

Shared memory interface

Frame handler

(MLX16)

Melexis LIN FW API

Signal Interaction layer

Standard LIN API

Application

LIN bus line

Interface to MSBI

Figure 2.1 LIN firmware layers

# Conventions and Type Definitions

All functions and types of the Melexis Standard LIN API specified in the LIN 2.0 LIN 2.1 LIN 2.2 specifications [1][2][3] have the prefix l_ (lowercase ‘L’ followed by an ‘underscore’) for the Core API functions, and the prefix ld_ for the Node Configuration and Identification functions.

The “static implementation” variant is used for the Standard LIN API functions using sl1 as the name of the interface. I.e. the interface management functions have the suffix _sl1.

## LIN Standard API data types

### Typedefs

| LIN Standard types | conventional types |
| --- | --- |
| l_bool | unsigned char |
| l_irqmask | unsigned int |
| l_u8 | unsigned char |
| l_u16 | unsigned int |
| l_u32 | unsigned long |
| l_u64 | unsigned long long |

### Enumerations

#### l_ifc_handle

Description

LIN interface determination

Enumerator

ifcSlave_1	Determine LIN Slave interface;

ifcMaster_1	Determine LIN Master interface.

#### SlaveResponse_t

Description

Slave responses.

Enumerator

respLinProductID 	answer to "Read by Identifier (0)" request

respSerialNumber 	answer to "Read by Identifier (1)" request

respNegativeResponse 	answer to non-supported sub-functions request

respAssignNAD 	answer to "Assign NAD" optional request

respConditionalChangeNAD 	answer to "Conditional change NAD" optional request

respPositiveAssignFidRange 	answer to "Assign Frame ID Range" request (LIN2.1, LIN2.2)

respUserDefIdCallout 	answer to user defined read_by_id request

### Data structures

#### l_s_IfcStatus_t

Description

LIN Status word.

Type

Union.

Data fields

l_u16 	word

Full ifc status word.

l_u8 	ErrorInResponse: 1

Error in response flag.

l_u8 	SuccessfulTrans: 1

Successful transfer flag.

l_u8 	Overrun: 1

Few frames were processed since the previous call.

l_u8 	GoToSleep: 1

Go to Sleep command was received.

l_u8 	BusActivity: 1

Node has detected activity on the bus.

l_u8 	EventTriggered: 1

Always 0 (zero) for Slave node.

l_u8 	SaveConfig: 1

Save configuration request was received.

l_u8 	PID: 8

Protected ID.

#### l_s_DiagFrame_t

Description

Structure of the Master Request and Slave Response frames. For more details please use the doxygen documentation.

## Constants

Default configuration constants and parameters such as baudrate, initial NAD, configured NAD and LIN Product ID should be generated from the LDF file [4]. Besides, constants for return values of the functions ld_set_configuration and ld_read_configuration should be defined.

# API Commands and events

## Driver and cluster management

### l_sys_init

Syntax

l_bool l_sys_init (void);

Description

Initializes the LIN Module framework.

Preconditions

The command shall be executed before any other LIN API’s command.

Return Value

Zero if the initialization succeeded;

Non-zero if the initialization failed.

Note

This is an empty function. The LIN Module, i.e. LIN interface, is initialized and configured in l_ifc_init.

Example

int main(void)

{

l_sys_init ();

l_ifc_init_sl1 ();

/* ...*/

}

## Signal interaction

Scalar signal writing and reading are atomic operations, i.e. new signal value is passed to the application when it is fully updated. This also applies to byte arrays. However, as per LIN specification [1][2][3], consistency is not guaranteed between any signals.

### Scalar signal read

Syntax

<signalType> l_<signalType>_rd_<signalName> (void);

Description

Reads and returns the current value of the signal with name <signalName> and of type <signalType>.

<signalType> can be any of the signal types specified in [1][2][3], i.e. l_bool, l_u8 and l_u16.

Return Value

The current value of the signal read, i.e. as last received by the LIN Module. The init_value (see Signal definition chapter of LDF specification [1][2][3]) of the signal is returned if the frame containing the signal has not yet been received.

Note

The actual functions should be generated from the LDF file; templates may be available to write these manually using C macros.

Example

for (;;) {

/* Copy signal sl1 to signal o1 (8 bits) */

if (l_flg_tst_s_sl1()) {

l_flg_clr_s_sl1();

tmp_u8 = l_u8_rd_sl1();

l_u8_wr_o1(tmp_u8);

}

}

### Scalar signal write

Syntax

void l_<signalType>_wr_<signalName> (<signalType> newValue);

Description

Sets the value of signal with name <signalName> and of type <signalType> to newValue. The init_value (see Signal definition chapter of LDF specification [1][2][3]) of the signal will be transmitted if the frame containing the signal is requested by Master and application has not changed the signal yet.

<signalType> can be any of the signal types specified in [1][2][3], i.e. l_bool, l_u8 and l_u16.

Return Value

None.

Note

The actual functions should be generated from the LDF file; templates may be available to write these manually using C macros.

### Byte array read

Syntax

void l_bytes_rd_<signalName> (l_u8 start,

l_u8 count,

l_u8 *const data);

Description

Reads the current values of the bytes start .. start+count-1 in the signal with name <signalName> and stored them in the pointed by data.

Precondition

The sum of start and count shall be lesser or equal to the length of the byte array.

The function shall be called while in protected mode, since may call l_sys_irq_xxx.

Return Value

None.

Note

The actual functions should be generated from the LDF file; templates may be available to write these manually using C macros.

Example

for (;;) {

if (l_flg_tst_f_byte_array_in()) {

l_flg_clr_f_byte_array_in();

/* Copy byte array "as is" */

l_bytes_rd_MstArray(0, 7, test_arr);

l_bytes_wr_SlvArray(0, 7, test_arr);

}

}

### Byte array write

Syntax

void l_bytes_wr_<signalName> (l_u8 start,

l_u8 count,

l_u8 const *const data);

Description

Sets the value of the bytes start .. start+count-1 signal with name <signalName> to the values stored at the locations pointed by data.

Precondition

The sum of start and count shall be lesser or equal to the length of the byte array.

The function shall be called while in protected mode, since may call l_sys_irq_xxx.

Return Value

None.

Note

The actual functions should be generated from the LDF file; templates may be available to write these manually using C macros.

## Notification

Flags are local objects in a node and they are used to synchronise the application program with the LIN core. The flags will be automatically set by the LIN core and can only be tested or cleared by the application program. Flags may be attached to all types of frames. A flag is set when the frame/signal is considered to be transmitted respectively received.

In Melexis implementation of Standard LIN API two types of flags are created:

A flag that is attached to a Signal

A flag that is attached to a Frame

The function prototypes to interact with flags are specified by Standard:

l_bool l_flg_tst_fff (void) - Returns the current state of the flag specified by the name fff: returns zero if the flag is cleared, non-zero otherwise

void l_flg_clr_fff (void) - Sets the current value of the flag specified by the name fff to zero

Flags can be attached to any signal and frame. Flag name consists of Signal / Frame name plus prefix "s_" for Signal and "f_" for Flag. For example, test flag function l_flg_tst_s_engineTemp(void) will be generated for Signal - "engineTemp".

### l_flg_tst

Syntax

l_bool l_flg_tst_<flagName> (void);

Description

Returns the current state of the flag with name <flagName>.

Return Value

Returns zero if the flag is cleared;

Returns non-zero if the flag is set.

Note

The actual functions should be generated from the LDF file using the signal and frame name as the flag name; templates may be available to write these manually using C macros.

### l_flg_clr

Syntax

void l_flg_clr_<flagName> (void);

Description

Clears the state of the flag with name <flagName>.

Return Value

None.

Note

The actual functions should be generated from the LDF file using the signal and frame name as the flag name; templates may be available to write these manually using C macros.

## Schedule management

Void. (LIN master only)

## Interface management

### Calls common for LIN 2.0, LIN 2.1 and LIN 2.2 API versions

#### l_ifc_init

Syntax for LIN2.1/LIN2.2

l_bool l_ifc_init_sl1 (void);

Syntax for LIN2.0/LIN1.3

l_bool l_ifc_init_sl1 (void);

Description

Initializes the single LIN Module, i.e. LIN interface.

Precondition

The command shall be executed before any other LIN API command, except l_sys_init.

Return Value

Zero if the initialization succeeded;

Non-zero if the initialization failed.

Note

The LIN API before version 2.1 does not have a return value [4];

The LIN2.1 version also implicitly connects the LIN Module to the bus.

See also

l_ifc_ioctl, ld_set_configuration

#### l_ifc_goto_sleep

Void. (LIN master only)

#### l_ifc_wake_up

Syntax

void l_ifc_wake_up_sl1 (void);

Description

This command will start the transmission of wake up sequence.

If there is no activity for 1.5 seconds after the third pulse, the LIN Module will enter the SLEEP state.

Precondition

The LIN Module should be in SLEEP or DISCONNECTED state.

Return Value

None.

Note

This is a wrapper around ml_WakeUp [4].

See also

ml_WakeUp [4]

#### l_ifc_ioctl

Note

Not yet implemented.

This is a wrapper around ml_SetBaudRate and ml_SetOptions. [4]

See also

ml_SetBaudRate, ml_SetOptions [4]

#### l_ifc_read_status

Syntax

l_u16 l_ifc_read_status_sl1 (void);

Description

This command returns the status of the previous communication.

The call is a read-reset call, meaning that the interface status word is set to 0 after the call has returned.

Return Value

The call returns the interface status word (16 bit value) as specified in [1][2][3] in the appropriate endianness:

Last frame Protected ID [15:8]

Save configuration [6:6] (LIN 2.1 only)

Event triggered frame collision  [5:5] (LIN 2.1 only; always 0 in the slave node)

Bus activity [4:4] (LIN 2.1 only)

Go to sleep [3:3]

Overrun  [2:2]

Successful transfer [1:1]

Error in response [0:0]

Note

Bus (in)activity is the only bit set not according to the frame transmitted or received by the node. It is checked immediately before returning the status word to the application, The bit is set, if the node has detected activity on the bus.

Atomic access to signals is guaranteed by entering system mode with highest priority.

### Calls specific for LIN 2.0 only

#### l_ifc_connect

Syntax

l_bool l_ifc_connect_sl1 (void);

Description

Connects the interface sl1 to the LIN cluster and enables the transmission of the data.

This command causes the LIN Module to switch from the DISCONNECTED state to the ACTIVE state and starts monitoring the LIN bus for activity.

Preconditions

This command can be used only from the DISCONNECTED state.

Return Value

Zero if the "connect operation" was successful

Non-zero if the "connect operation" failed.

Note

This is a wrapper around ml_Connect. [4]

See also

ml_Connect [4]

#### l_ifc_disconnect

Syntax

l_bool l_ifc_disconnect_sl1 (void);

Description

Disconnects the interface sl1 from the LIN cluster and thus disables the interaction between this and other nodes in the cluster.

Return Value

Zero if the "disconnect operation" was successful

Non-zero if the "disconnect operation" failed.

Note

This is a wrapper around ml_Disconnect. [4]

See also

ml_Disconnect [4]

### Calls specific for Melexis LIN API implementation

#### l_ifc_sleep_entered_callout

Syntax

void l_ifc_sleep_entered_callout (ml_StateReason Reason);

Description

Call-back function which is called by the LIN driver to report a reason for entering LIN sleep state.

If application specific implementation of l_ifc_sleep_entered_callout is not provided then default (empty) implementation from the LIN API is used. In the default implementation all reported reasons are ignored.

Possible reason which might be reported (see [4] for details):

ml_reasonMaster: go-to-sleep frame is received from the LIN master

ml_reasonCommand: sleep command is received from the application.

ml_reasonTimeOut: LIN bus timeout (> 4 seconds) with LIN bus in recessive state

ml_reasonTimeOutDominant: LIN bus timeout (>4 seconds with LIN bus in dominant state

ml_reasonWakeup: wake-up pulse is detected in on the LIN bus.

ml_reasonWakeupResponse: a response to wake-up pulse is detected

ml_reasonWakeupAbort: no bus activity after initiating the wakeup sequence

Precondition

None

Return Value

None

## User provided call-outs

### l_sys_irq_disable

Syntax

l_irqmask l_sys_irq_disable (void);

Description

Disables all interrupts.

Return Value

Returns the CPU priority level before the function call. To be used in a subsequent l_sys_irq_restore call.

### l_sys_irq_restore

Syntax

void l_sys_irq_restore (l_irqmask previous);

Description

Restores CPU priority level to previous.

Return Value

None.

## Network configuration and identification

### Calls common for LIN 2.0, LIN 2.1 and LIN 2.2 API versions

#### ld_conditional change_NAD

Void. (LIN master only)

Note

Slave behavior upon reception of this node configuration and diagnostic frame is implemented in generic implementation of the mlu_MessageReceived [4] notification.

This service is optional in the network.

See also

ml_DiagMasterRequest

#### ld_read_by_id

Void. (LIN master only)

Note

Slave behavior upon reception of this node configuration and diagnostic frame is implemented in generic implementation of the mlu_MessageReceived [4] notification.

See also

ml_DiagMasterRequest

#### ld_assign_nad

Void. (LIN master only)

Note

Slave behavior upon reception of this node configuration and diagnostic frame is implemented in generic implementation of the mlu_MessageReceived [4] notification.

See also

ml_DiagMasterRequest

#### ld_read_by_id_callout

Syntax

l_u8 ld_read_by_id_callout (l_ifc_handle iii, l_u8 id, l_u8 *pci, l_u8 *data)

Description

This callout function is called by the LIN driver to retrieve data from the user application. The data is requested by the LIN driver when preparing a response to master’s “Read by identifier” request with the provided parameter id.

The id parameter is the identifier in the user defined range (a value in the range of 32 to 63 inclusive) from the “Read by identifier” request.

The data array up to 5 bytes must be used by the application to set up the positive response. The pci defines the data length plus one.

Preconditions

This callout function is called by the LIN driver when a “Read by identifier” request with an identifier in the range of 32 to 63 inclusive is received from the master.

This callout function may also be called by the LIN driver in case an identifier in this range is used in Conditional Change NAD request received from the master.

Enabled by HAS_READ_BY_ID_CALLOUT define in lin_signals.h.

Return Value

LD_NEGATIVE_RESPONSE - The slave node will respond with a negative response. In this case data is not considered.

LD_POSITIVE_RESPONSE - The slave node will setup a positive response using the data provided by the application.

LD_NO_RESPONSE - The slave node will not answer.

Note

The function interface differs from the Standard definition because of additional pci parameter.

The driver will act according to the return values from the application.

This service is optional in the network and the callout is available in slave node only. In case the user defined read by identifier request is used, the slave node application must implement this callout.

This callout function is only defined in the standard LIN API since LIN 2.1 [4].

Example

l_u8 ld_read_by_id_callout (l_ifc_handle iii, l_u8 id, l_u8* data)

{

(void) iii;

/*

* id values are in range from 0x20 to 0x3F (32 to 63),

* their meanings are application dependent

*/

if (id == 0x20)

{

*pci=0x06;

data[0] = 0xAA;

data[1] = 0xBB;

data[2] = 0xCC;

data[3] = 0xDD;

data[4] = 0xEE;

return LD_POSITIVE_RESPONSE;

}

else

{

return LD_NEGATIVE_RESPONSE;

}

}

See also

ld_serial_number_callout

### Calls specific for LIN 2.0 only

#### ld_is_ready

Void. (LIN master only)

#### ld_check_response

Void. (LIN master only)

#### ld_assign_frame_id

Void. (LIN master only)

Note

Slave behavior upon reception of this node configuration and diagnostic frame is implemented in generic implementation of the mlu_MessageReceived [4] notification.

See also

ml_DiagMasterRequest

### Calls specific for LIN 2.1 only

#### ld_is_ready

Void. (LIN master only)

#### ld_check_response

Void. (LIN master only)

#### ld_assign_frame_id_range

Void. (LIN master only)

Description

This call assigns the protected identifier of up to four frames in the slave node with the addressed  NAD. The PIDs parameter shall be four bytes long, each byte shall contain a PID, do not care or unassign value.

#### ld_save_configuration

Void. (LIN master only)

Note

Slave behavior upon reception of this node configuration and diagnostic frame is implemented in generic implementation of the mlu_MessageReceived [4] notification.

This service is optional in the network.

See also

ml_DiagMasterRequest

#### ld_read_configuration

Syntax

l_u8 ld_read_configuration (l_ifc_handle iii, l_u8* const data, l_u8* const length);

Note

This function will not yield any communication on the LIN bus.

The function will serialize the current configuration (NAD, PIDs) and copy it to the area (data pointer) provided by the application. The intention is to call this function when the save configuration request flag is set in the interface status word, whereas the application is responsible to save this data in non-volatile memory.

Preconditions

Enabled by HAS_SAVE_CONFIGURATION_SERVICE define in lin_signals.h.

Return Value

LD_READ_OK – If the service was successful.

LD_LENGTH_TOO_SHORT – If the configuration size is greater than the length. It means that the data area does not contain a valid configuration.

See also

l_ifc_read_status, ld_set_configuration

#### ld_set_configuration

Syntax

l_u8 ld_set_configuration (l_ifc_handle iii, l_u8* const data, l_u16 length);

Note

This function will not yield any communication on the LIN bus.

The function will configure the NAD and the PIDs according to the configuration given by data. The intended usage is to restore a saved configuration or to set an initial configuration.

Preconditions

Enabled by HAS_SAVE_CONFIGURATION_SERVICE define in lin_signals.h.

Return Value

LD_SET_OK – If the service was successful.

LD_LENGTH_NOT_CORRECT – If the required size of the configuration is not equal to the given length.

LD_DATA_ERROR – The set of configuration could not be made.

See also

ld_read_configuration, l_ifc_read_status

### Calls specific for Melexis LIN API implementation

#### ml_DiagClearPendingResponse

Syntax

void ml_DiagClearPendingResponse (void);

Description

This function clears the pending slave's response (if any) to be sent with a subsequent Slave response frame.

Preconditions

This function should be called by an application to inform the LIN driver to abort a pending network management, transport layer or diagnostics communication. For example in case of reception of a corrupted master request frame, or upon N_As and N_Cr timeout. [4].

Return Value

None.

Note

Since the standard LIN API [1][2][3] does not provide any callout function to inform the driver of an ongoing communication, and to support any application software architecture, this non-standard ml_DiagClearPendingResponse function is introduced.

See also

mlu_TL_TimeoutStart

#### ld_serial_number_callout

Syntax

void ld_serial_number_callout (l_u8 data[4]);

Description

This callout function is called by the LIN driver to retrieve the serial number from the user application. The serial number is requested by the LIN driver when preparing a response to master’s “Read by identifier” request with id=1 (serial number).

The data array of 4 bytes must be used by the user application to transfer the serial number.

Preconditions

This callout function is called by the LIN driver when a “Read by identifier” request with id=1 (serial number) is received from the master.

This callout function may also be called by the LIN driver in case the “serial number” identifier is used in Conditional Change NAD request received from the master.

Enabled by HAS_SERIAL_NUMBER_CALLOUT define in lin_signals.h.

Return Value

None.

Note

Since the standard LIN API [1][2][3] does not provide any callout function to request serial number from user application, this non-standard ld_serial_number_callout function is introduced.

This service is optional in the network and the callout is available in slave node only. In case the serial number read by identifier request is used, i.e. id 1, the slave node application must implement this callout.

Example

void ld_serial_number_callout (l_u8 data[4])

{

data[0] = (l_u8)ML_SERIAL_NUMBER;           /* LSB */

data[1] = (l_u8)(ML_SERIAL_NUMBER >> 8);

data[2] = (l_u8)(ML_SERIAL_NUMBER >> 16);

data[3] = (l_u8)(ML_SERIAL_NUMBER >> 24);   /* MSB */

}

See also

ld_read_by_id, ld_conditional_change_NAD, ld_read_by_id_callout

# Disclaimer

Devices sold by Melexis are covered by the warranty and patent indemnification provisions appearing in its Term of Sale. Melexis makes no warranty, express, statutory, implied, or by description regarding the information set forth herein or regarding the freedom of the described devices from patent infringement. Melexis reserves the right to change specifications and prices at any time and without notice. Therefore, prior to designing this product into a system, it is necessary to check with Melexis for current information. This product is intended for use in normal commercial applications. Applications requiring extended temperature range, unusual environmental requirements, or high reliability applications, such as military, medical life-support or life-sustaining equipment are specifically not recommended without additional processing by Melexis for each application. The information furnished by Melexis is believed to be correct and accurate. However, Melexis shall not be liable to recipient or any third party for any damages, including but not limited to personal injury, property damage, loss of profits, loss of use, interrupt of business or indirect, special incidental or consequential damages, of any kind, in connection with or arising out of the furnishing, performance or use of the technical data herein. No obligation or liability to recipient or any third party shall arise or flow out of Melexis’ rendering of technical or other services.

© 2016 Melexis NV. All rights reserved.